Splash Screen Text

Product Information

These data provide an estimate of potential coastal shoreline change (e.g., erosion) in response to sea level rise during the 21st century, to help identify, assess, and plan for possible areas of vulnerability. The shoreline position is defined as the land-water boundary at the Mean High Water (MHW) line determined at set transects.

The projections are made from a model that shows change in shoreline position along cross-shore transects, considering water level, wave conditions, and along-shore and cross-shore sediment transport. Complex coastal processes are included as parameters in the model and run in an ensemble (hundreds of simulations) to obtain a robust estimate of change. Illustrated shorelines may look ‘spikey’ in select locations—this is the result of the information being shown at the resolution of individual transects. For detailed questions, please refer to metadata. 

Modeled Scenarios

Projections of shoreline change were made for several SLR scenarios, from 25 cm (0.8 ft) to 500 cm (16.4 ft). SLR of 0 m is reflective of conditions in 2005, to be consistent with SLR projections from the National Oceanic and Atmospheric Administration (NOAA). To highlight potential trends, the initial model shoreline (automatically derived from satellite imagery) is included. This shoreline should not be used in lieu of definitive historical shoreline records. 

To better support coastal decision making, projections were made for different model cases, representing different assumptions on key factors affecting shoreline change. These factors included (1) whether the shoreline evolved past current landward boundaries (e.g., vegetation, bluffs, infrastructure, armoring) separating the shore from upslope landscape types, and (2) whether historical indicators of accretion, which in some areas results from management interventions like beach nourishment, were accounted for in model parameters (see metadata for more details). These two factors were combined for four different model cases. In this dataset, [INSERT LANGUAGE FOR CORRESPONDING CASE FROM BELOW]

CASE 1 (hold_the_line/no_nourishment): (1) a “Landward Model Boundary” for the future shoreline position is imposed, assuming that existing infrastructure or other features (e.g., cliff, dune toe) will limit the extent of future erosion, and (2) the model assumes recent historical accretion rates DO NOT continue into the future.

CASE 2 (hold_the_line/continued_nourishment): (1) a “Landward Model Boundary” for the future shoreline position is imposed, assuming that existing infrastructure or other features (e.g., cliff, dune toe) will limit the extent of future erosion, and (2) the model assumes recent historical accretion rates CONTINUE into the future.

CASE 3 (no_hold_the_line/no_nourishment): (1) shoreline positions along each transect are allowed to move landward WITHOUT LIMITATION. Infrastructure or other features (e.g., cliff, dune toe) are NOT included in this model case, and no assumptions about these features are made; and (2) the model assumes recent historical accretion rates DO NOT continue into the future.

CASE 4 (no_hold_the_line/continued_nourishment): (1) shoreline positions along each transect are allowed to move landward WITHOUT LIMITATION. Infrastructure or other features (e.g., cliff, dune toe) are NOT included in this model case, and no assumptions about these features are made; and (2) the model assumes recent historical accretion rates CONTINUE into the future.

Uncertainty

Uncertainty should be considered in any vulnerability assessment, particularly when evaluating exposure risk for critical assets. “Model uncertainty” is shown as a yellow band for each transect. It represents the 95 percent confidence bands associated with a 200-member ensemble of modeled shoreline positions, representing robust model uncertainty (see metadata for details). As bands narrow, it can help indicate where there is higher confidence in the projected changes. These bands also take seasonal variations in the shoreline into account. 

Shoreline changes and erosion from extreme storms can lie outside the 95% confidence interval. To illustrate the potential episodic impact of extreme storms, the maximum landward shoreline deviation for wave heights of certain intensities (return periods of 1-year, 20-year and 100-years) are also provided as “potential storm erosion uncertainty” to illustrate the potential additional impact of extreme events.

Lastly, while several processes are implicitly included as noise parameters for each transect location, the model does not explicitly account for all coastal processes. In highly dynamic and complex areas such as river mouths, capes, inlets, and end of spits, uncertainty is greater. Thus, an estimate of potential uncertainty for SLR scenarios 25cm and greater is included for “unresolved processes” and other sources of error not explicitly included in the model.

Amplifying information for specific variables

Initial shoreline

Position of initial shoreline used at the start of model simulation. Derived from satellite imagery dated closest to 01 January 1995. The position is included only to highlight trends in projected shoreline change and should not be used in place of definitive historical shoreline records. The shoreline position is defined as the land-water boundary at the Mean High Water (MHW) line determined at set transects.

Modeled shoreline

Position of shoreline, defined as the land-water boundary at the Mean High Water (MHW) line for the indicated SLR scenario. This represents the mean trajectory of a 200-member ensemble using multiple realizations of future wave conditions. As the models use the change in sea level over time during computation, final shoreline positions for SLR scenarios are identified for the date indicated.

Modeled shoreline uncertainty

Modeled uncertainty for projected shoreline position for the given SLR scenario, defined as the upper and lower bounds of a 95% confidence interval for the projected 200-member ensemble trajectories.  It is representative of long-term, seasonal, and daily shoreline change.

Potential storm erosion uncertainty

Illustrates the potential additional impact of an extreme storm on episodic shoreline change. It is defined as the maximum modeled shoreline position from the 200-member ensemble for wave heights of the storm return-period/intensity indicated. Thus, it is the maximum potential erosion trajectory from the model ensemble.

Unresolved process uncertainty

Uncertainty considering unresolved processes and other sources of error not explicitly included in the model, which captures the larger uncertainty in model projections for dynamic and complex areas such as river mouths, capes, inlets, and the ends of spits. Estimates of the process uncertainty are derived from comparing model output with Satellite-derived Shoreline (SDS) observations during the validation period (2015-2020). Unresolved process uncertainty is shown for SLR scenarios 25 cm and greater.

Transects

Identification of the cross-shore transect for each model location, unique across study area.

Transects are colored by what type of model was run for that transects: “full model” configuration included all model parameters; “cross-shore only” configuration excluded longshore transport in locations where this was applicable (for example, when the beach was very short or enclosed); and a “rate only” configuration shows where only historical rates of shoreline change are used. As the governing equations in this shoreline model are applicable for responses of a sandy beach, model parameters would not be appropriately descriptive for very cobbled beaches (or similarly heterogeneous beaches) and a more simplified approach of historical rates was used.  See metadata and references for more information.

Landward Model Boundary

This boundary is used in specific model cases where shoreline change is restricted and not allowed to erode further landward. It is the location along individual transects that denotes the landward end of the beach during model simulations, defined as the boundary between the beach and upslope landscapes, such as vegetated or urban landscapes. It represents a scenario where existing infrastructure or other features (e.g., cliff, dune toe) may limit the extent of future erosion. For contrast and comparison, other model cases show projections where the shoreline is allowed to erode without restriction. See metadata for more explanation on the model cases.

Shoreline change hazard zone

Comprehensive hazard zone denoting area impacted by projected shoreline change, defined as the area between the initial shoreline and the modeled shoreline uncertainty. Modeled shoreline uncertainty represents the 95% confidence interval for the projected 200-member ensemble trajectories, and is representative of long-term, seasonal, and daily shoreline change. Anything within this hazard zone is likely to be impacted by shoreline change for the SLR indicated.

Extreme storm shoreline change hazard zone

Comprehensive hazard zone denoting area impacted by projected shoreline change, including potential additional impact of an extreme storm on episodic shoreline change. It is defined as the area between the initial shoreline and the potential storm erosion uncertainty for the 100-year return-period/intensity storm. Anything within this hazard zone is likely to be impacted by shoreline change for the SLR indicated as well as episodic shoreline change from an extreme storm.