Splash Screen Text

Product Information

These data provide an estimate of potential coastal shoreline change (e.g., erosion) in response to sea-level rise (SLR) during the 21st century, to help identify, assess, and plan for possible areas of vulnerability. The shoreline position is defined as the land-water boundary at Mean Sea Level (MSL), determined at set transects.

[NEWLINE]

The projections are made with the CoSMoS-COAST model, that computes change in shoreline position along cross-shore transects, considering water level, wave conditions, and along-shore and cross-shore sediment transport. Complex coastal processes are included as parameters in the model and run in an ensemble (hundreds of simulations) to obtain a robust estimate of change. Illustrated shorelines may look ‘spikey’ in select locations—this is the result of the information being shown at the resolution of individual transects. For detailed questions, please refer to metadata. 
[NEWLINE]

Model Scenarios

Projections of shoreline change were made for several SLR scenarios, from 25 cm (0.8 ft) to [INSERT TEXT 1]. SLR of 0 cm is reflective of conditions in the year [INSERT TEXT 2]. To highlight trends in projected vs. historical shoreline position, the initial model shoreline (automatically derived from satellite imagery) is included. This shoreline should not be used in lieu of definitive historical shoreline records (e.g., from Lidar).

[NEWLINE]

TEXT 1 CALIFORNIA: 500 cm (16.4 ft)

TEXT 1 FLOSUP: 300 cm (9.8 ft)

TEXT 2 CALIFORNIA: 2000

TEXT 2 FLOSUP: 2005, to be consistent with the latest SLR projections from the Federal Interagency Task Force (Sweet et al. 2022).

To better support coastal decision making, projections were made for different model cases, representing different assumptions on key factors affecting shoreline change. These factors included (1) whether the shoreline evolved past current landward boundaries (e.g., vegetation, bluffs, infrastructure, armoring) separating the shore from upslope landscape types, and (2) whether historical indicators of accretion, which in some areas results from management interventions like beach nourishment, were accounted for in model parameters (see metadata for more details). These two factors were combined for four different model cases. In this dataset, [INSERT LANGUAGE FOR CORRESPONDING CASE FROM BELOW]

[NEWLINE]

CASE 1 (hold_the_line/no_nourishment): (1) a “Landward Model Boundary” for the future shoreline position is imposed, assuming that existing infrastructure or other features (e.g., cliff, dune toe) will limit the extent of future erosion, and (2) the model assumes recent historical accretion rates DO NOT continue into the future.

CASE 2 (hold_the_line/continued_nourishment): (1) a “Landward Model Boundary” for the future shoreline position is imposed, assuming that existing infrastructure or other features (e.g., cliff, dune toe) will limit the extent of future erosion, and (2) the model assumes recent historical accretion rates CONTINUE into the future.

CASE 3 (no_hold_the_line/no_nourishment): (1) shoreline positions along each transect are allowed to move landward WITHOUT LIMITATION. Infrastructure or other features (e.g., cliff, dune toe) are NOT included in this model case, and no assumptions about these features are made; and (2) the model assumes recent historical accretion rates DO NOT continue into the future.

CASE 4 (no_hold_the_line/continued_nourishment): (1) shoreline positions along each transect are allowed to move landward WITHOUT LIMITATION. Infrastructure or other features (e.g., cliff, dune toe) are NOT included in this model case, and no assumptions about these features are made; and (2) the model assumes recent historical accretion rates CONTINUE into the future.

Model Uncertainty

Uncertainty is an important component of vulnerability assessments, particularly when evaluating exposure risk for critical assets. “Model uncertainty” is shown as yellow bands for each transect, representing the 95 percent confidence bands associated with a 200-member ensemble of modeled shoreline positions (see metadata for details). Narrow uncertainty bands indicate where there is higher confidence in the projected changes. These bands also account for seasonal variations in the shoreline.

[NEWLINE]

Shoreline changes and erosion from extreme storms can lie outside the 95% confidence interval. To illustrate the potential episodic impact of extreme storms, the maximum landward shoreline deviation for extreme erosion events of certain intensities (return periods of 1-year, 20-year and 100-years) are also provided and referred to as “potential storm erosion uncertainty” to illustrate the potential additional impact of extreme events.

[NEWLINE]

Lastly, while several processes are implicitly included as noise parameters for each transect location, the model does not explicitly account for all coastal processes. In highly dynamic and complex areas, such as river mouths, capes, inlets, and end of spits, uncertainty is often greater. Thus, an estimate of potential uncertainty for SLR scenarios 25cm and greater is included for “unresolved processes” and other sources of error not explicitly included in the model.

Amplifying information for specific variables (these are the folder-level popups)

Initial shoreline

Position of the initial shoreline used at the start of model simulation, which is derived from satellite imagery dated closest to the model start time (ca. 01 January 1995). The position is included to highlight trends in projected vs. historical shoreline position and should not be used in place of definitive historical shoreline records (e.g., from Lidar). The shoreline position is defined as the land-water boundary at Mean Sea Level (MSL), determined at set transects.

Modeled shoreline

Position of shoreline, defined as the land-water boundary at Mean Sea Level (MSL) for the indicated SLR scenario. This represents the median trajectory of a 200-member ensemble using multiple realizations of future wave conditions. As the models use the change in sea level over time during computation, final shoreline positions for SLR scenarios are identified for the date indicated.

Modeled shoreline uncertainty

Modeled uncertainty for projected shoreline position for the given SLR scenario, defined as the upper and lower bounds of a 95% confidence interval for the projected 200-member ensemble trajectories.  It is representative of long-term, seasonal, and daily shoreline change. In model cases where a landward model boundary was imposed (cases 1 and 2), uncertainty does not extend past the model boundary.

Potential storm erosion uncertainty

Illustrates the potential additional impact of extreme storms on episodic shoreline change for full-model and cross-shore-only transects. It is defined from an extreme value analysis of modeled shoreline positions from the 200-member ensemble based on the return-period/intensity indicated. Thus, it captures extreme potential erosion events from the model ensemble. For other model configurations (e.g., ‘rate only’), see metadata for full explanation.

Unresolved process uncertainty

Uncertainty considering unresolved processes and other sources of error not explicitly included in the model, which captures the larger uncertainty in model projections for dynamic and complex areas such as river mouths, capes, inlets, and the ends of spits. Estimates of the so-called unresolved process uncertainty are derived from comparing model output with Satellite-derived Shoreline (SDS) observations during the validation period (2015-2020). Unresolved process uncertainty is shown for SLR scenarios 25 cm and greater and where validation data were present.

Transects

Identification of the cross-shore transect for each model location, unique across study area.

Transects are colored by what type of model was run for that transects: “full model” configuration included all model parameters; “cross-shore only” configuration excluded longshore transport in locations where this was applicable (for example, when the beach was short or enclosed); and a “rate only” configuration shows where only historical rates of shoreline change are used. As the governing equations in this shoreline model are primarily applicable for responses of a sandy beach, model parameters may not be appropriately descriptive for very cobbled beaches (or similarly heterogeneous beaches) and a more simplified approach of historical rates was used.  See metadata and references for more information.

Landward Model Boundary

This boundary is used in specific model cases where shoreline change is restricted and not allowed to erode further landward. It is the location along individual model transects that denotes the landward end of the beach, defined as the boundary between the beach and upslope landscapes, such as vegetated or urban landscapes. This boundary is used as part of model cases where existing infrastructure or other features (e.g., cliff, dune toe) may limit the extent of future erosion. For contrast and comparison, other model cases show projections where the shoreline is allowed to erode without restriction. See metadata for more explanation on the model cases.

Shoreline change hazard zone

Comprehensive hazard zone denoting areas impacted by projected shoreline change, defined as the area between the initial shoreline and the modeled shoreline uncertainty. Modeled shoreline uncertainty represents the 95% confidence interval for the projected 200-member ensemble, and is representative of long-term, seasonal, and daily shoreline change. Anything within this hazard zone is likely to be impacted by shoreline change for the SLR indicated.

Extreme storm hazard zone

Comprehensive hazard zone denoting area impacted by projected shoreline change, including potential additional impact of an extreme storm on episodic shoreline change. It is defined as the area between the initial shoreline and the potential storm erosion uncertainty for 100-year return-period/intensity erosion events. Anything within this hazard zone is likely to be impacted by shoreline change for the SLR indicated as well as episodic shoreline change due to extreme storm events.

Definitions for lines/polygons/points

### initial shoreline

geographic position of the initial shoreline used in the model, selected from shoreline observations circa the model start time

### modeled shoreline

modeled shoreline position for the SLR indicated (model projection/hindcast [INSERT DATE])

### upper bound modeled shoreline uncertainty

modeled uncertainty of shoreline change determined at the 95% confidence interval for SLR indicated (model projection/hindcast [INSERT DATE]).

### lower bound modeled shoreline uncertainty

modeled uncertainty of shoreline change determined at the 95% confidence interval for SLR indicated (model projection/hindcast [INSERT DATE]).

### potential storm erosion uncertainty (NNNN-yr storm)

uncertainty in modeled shoreline position considering potential storm erosion for SLR and storm intensity for given return period indicated (model projection/hindcast [INSERT DATE]).

### Landward Model Boundary (point name is Transect#) -- [SLR_cm is NA]

landward extent of the beach used in the model simulations along this transect

### shoreline change hazard zone

shoreline change hazard zone for SLR indicated (model projection/hindcast [INSERT DATE])

### extreme storm hazard zone

shoreline change hazard zone including extreme storm impact for SLR indicated (model projection/hindcast [INSERT DATE])

### unresolved process uncertainty

uncertainty associated with unresolved processes and other sources of error not explicitly included in the model for the SLR indicated (model projection/hindcast [INSERT DATE])